| S.No | Description of Item | Qty | Unit | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| A | CIVIL WORKS |  |  |  |  |
| 1 | Surface preparation and painting of external walls including scraping, removing loose plaster/paint, cleaning fungal/algae growth, crack opening & filling with approved exterior crack sealant with: • 1 coat exterior primer (alkali-resistant) • 2 coats premium exterior emulsion (Weatherproof grade – Asian Apex/ Nerolac Excel/ Berger Weathercoat or equivalent). Work include scaffolding or other works necessary for painting of 3mtr boundary wall / 8 mtr high building wall as per direcion of EIC. | 18900 | SQFT |  |  |
| 2 | Interior wall surface preparation and painting including removing loose paint/plaster, sanding & levelling with 1 coat primer + 2 coats washable interior emulsion paint. Work include scaffolding or other works necessary for painting of 3.5 mtr high building wall as per direcion of EIC. | 38500 | SQFT |  |  |
| 3 | Re-plastering damaged wall areas including dismantling with 10-15 mm thick cement mortar 1:4/1:5 including curing (patch repairs) | 1500 | SQFT |  |  |
| 4 | Ceiling preparation & painting including scraping, remove loose paint/plaster, cleaning fungal/algae growth with 1 coat of primer + 2 coats washable interior emulsion. Work include scaffolding or other works necessary for painting of internal ceiling as per direcion of EIC. | 10600 | SQFT |  |  |
| 5 | Painting of MS grill/ collapsible gate/ steel items with 1 coat metal primer + 2 coats enamel paint | 1750 | SQFT |  |  |
| B | TERRACE WATERPROOFING |  |  |  |  |
| 6 | PCC protective screed (5-6 mm) 1:2:4 mix with waterproofing compound ensuring proper slope. Also, includes surface cleaning & preparation at terrace (removing loose coating, dust, laitance, etc.) | 5500 | SQFT |  |  |
| 7 | Waterproofing treatment using polymer modified cementitious waterproof coating (minimum 2–3 coats of Dr. Fixit Dampguard or SikaTop Seal or Asian Paints SmartCare Damp Proof) over entire terrace area, including parapet junction sealing | 5500 | SQFT |  |  |
| C | WASHROOM REPAIRS |  |  |  |  |
| 8 | Replacement of damaged sanitary fixtures (cisterns, taps, basins etc.) with new fixtures including all accessories, etc. as per direction of EIC. |  |  |  |  |
| a | European Water Closet | 2 | nos |  |  |
| b | Wash Basin | 2 | nos |  |  |
| c | Cistern | 2 | nos |  |  |
| d | Taps/cock - all type | 4 | nos |  |  |
| e | Drain pipe | 3 | nos |  |  |
| D | WOODWORK & TERMITE TREATMENT |  |  |  |  |
| 9 | Anti-termite treatment for doors/frames (chemical injection + brushing). Repair of wooden frames/doors, sanding & surface finishing. | 6 | set |  |  |
| 10 | Painting/ varnishing of woodwork with primer + 2 coats synthetic enamel/ PU polish over door frames/doors, etc. as per direction of EIC. | 800 | sqft |  |  |
| F | SITE CLEANING & DISPOSAL |  |  |  |  |
| 11 | Disposal of debris, site clearance and handover the premises in good condition. Washroom complete and all floors (inside and outside both) deep cleaning & disinfection after works. | 1 | LS |  |  |
|  |  |  |  | Total = |  |

Above rates and amounts are excluding GST and GST as applicable shall be charged extra.